ANTIDOTE

Installation Guide  |  v1.7  |  From Bare Parts

# 1. Bill of Materials

| Component | Model | Notes |
| --- | --- | --- |
| Single-board computer | Raspberry Pi Zero 2W | Running Kali Linux. Requires powered USB hub hat for 3 USB dongles. |
| WiFi dongle x2 | Panda PAU0A AC600 | MT7610U chipset. One for capture, one for AP/inject |
| BLE dongle | Laird BL654 (nRF52840) | Flashed with Zephyr hci_usb firmware |
| Microcontroller | Seeed XIAO ESP32-S3 | Connected via UART (GPIO14/15) and USB |
| MicroSD card | 16GB+ Class 10 | For Kali Linux OS |
| Power supply | 5V 2.5A micro-USB | Pi Zero 2W uses micro-USB power (PWR port). Hub hat may supply power independently — verify your hat’s power path before connecting both. |

# 2. Interface Assignment

| Hardware | Interface | Role | Direction | Description |
| --- | --- | --- | --- | --- |
| Pi Built-in BCM43430 | wlan0 | Idle / Future WiFi uplink | — | Reserved for wireless internet access. Not used for capture or injection. |
| Panda PAU0A #1 (MT7610U) | wlan1 / wlan1mon | WiFi Capture (Passive) | INBOUND | Monitor mode via iw. Passively captures probe requests, beacon SSIDs, and BSSIDs from nearby devices. |
| Panda PAU0A #2 (MT7610U) | wlan2 | SSID Beacon Injection + Management AP | OUTBOUND / MGMT | Injects spoofed beacon frames from pool SSIDs. Also hosts management AP (hostapd) for wireless web UI access. |
| Laird BL654 nRF52840 | hci0 | BLE MAC Spoofing | OUTBOUND | Cycles random MAC address via HCI LE Set Random Address. Re-advertises pool BLE MACs to confuse passive BLE scanners. |
| XIAO ESP32-S3 | /dev/serial0 + /dev/ttyACM0 | BLE Exhale (Advertising) | OUTBOUND | Receives pool samples via UART. Advertises BLE identifiers using onboard radio. Drives RGB LED status indicator. |

wlan0 (Pi built-in BCM43430) cannot use monitor mode on kernel 6.12.x. All capture uses wlan1 (PAU0A #1, MT7610U) via iw monitor mode.

# 3. Raspberry Pi Setup

## 3.1 Install Kali Linux

- Download Kali Linux for Raspberry Pi from kali.org/get-kali

- Flash to MicroSD using Raspberry Pi Imager or balenaEtcher

- Boot Pi, complete initial setup, update: sudo apt update && sudo apt full-upgrade -y

## 3.2 Install System Dependencies

sudo apt install -y python3-pip python3-venv aircrack-ng hostapd dnsmasq

sudo apt install -y bluetooth bluez libbluetooth-dev

sudo apt install -y git iw wireless-tools usb-modeswitch

Prevent hostapd and dnsmasq from starting as system services (Antidote manages them):

sudo systemctl disable hostapd dnsmasq

sudo systemctl stop hostapd dnsmasq

## 3.3 Create Python Virtual Environment

Pi Zero 2W only — increase swap before pip install. The Zero 2W has 512 MB RAM; bleak’s dbus-fast dependency requires Cython compilation that exhausts RAM without swap:

sudo dphys-swapfile swapoff

sudo sed -i 's/CONF_SWAPSIZE=.*/CONF_SWAPSIZE=1024/' /etc/dphys-swapfile

sudo dphys-swapfile setup && sudo dphys-swapfile swapon

After pip installs complete you may reduce swap back to 100 MB if desired. Pi 4B users can skip this step.

Create the virtual environment and install Python dependencies:

python3 -m venv /home/kali/antidote-env

source /home/kali/antidote-env/bin/activate

pip install scapy flask pyserial bleak

## 3.4 USB Port Layout

| USB Port | Device | Notes |
| --- | --- | --- |
| Hub port 1 | PAU0A #1 (wlan1) | WiFi capture dongle |
| Hub port 2 | PAU0A #2 (wlan2) | AP / injection dongle |
| Hub port 3 | XIAO ESP32-S3 | USB black port on XIAO |
| Hub port 4 | Laird BL654 (hci0) | nRF52840 BLE dongle |

Pi Zero 2W USB/OTG note: The Zero 2W has a single micro-USB OTG data port. All three USB dongles (PAU0A #1, PAU0A #2, Laird BL654) connect via a powered USB hub hat on that port. Verify your hub hat provides data passthrough (not power-only) and sufficient current — the two MT7610U adapters draw up to 500mA each under load; the Zero 2W itself draws ~300mA; total budget is 1.2–1.5A minimum. The wlan0 built-in radio is used only for the management AP; it is not involved in capture or injection.

# 4. nRF52840 / Laird BL654 Firmware

## 4.1 Build Zephyr hci_usb Firmware (Windows)

- Install nRF Connect SDK 2.9.2 via nRF Connect for Desktop Toolchain Manager

- Open Toolchain Manager, click the arrow next to v2.9.2, select Open Terminal

- In the terminal, navigate to the Zephyr directory:

cd C:\ncs\v2.9.2\zephyr

- Build the hci_usb sample:

west build -p always -b nrf52840dongle/nrf52840 samples/bluetooth/hci_usb

- Output firmware: build\hci_usb\zephyr\merged.hex

## 4.2 Flash Firmware

- Hold the reset button on the BL654 dongle to enter DFU mode (LED blinks red)

- Open nRF Connect Programmer

- Select the dongle device (appears as BOOTLOADER)

- Add HEX file: select merged.hex from the build output

- Click Write — wait for completion

- Dongle reboots and enumerates as USB HCI device (VID:PID 2fe3:000b)

## 4.3 Verify on Pi

hciconfig

Should show hci0 with USB transport. If not visible:

sudo rfkill unblock bluetooth

sudo hciconfig hci0 up

hciconfig shows 00:00:00:00:00:00 for the public address — this is normal for the nRF52840 in hci_usb mode. Use bluetoothctl show to verify the device is present.

# 5. ESP32-S3 Setup

## 5.1 Flash MicroPython

- Download MicroPython for ESP32-S3 from micropython.org

- Flash using esptool: esptool.py --chip esp32s3 --port /dev/ttyACM0 write_flash 0 firmware.bin

## 5.2 Deploy ESP32 Code

- Extract esp32/ folder from the release zip

- Copy files to ESP32 using mpremote or Thonny:

mpremote connect /dev/ttyACM0 cp main.py :main.py

mpremote connect /dev/ttyACM0 cp uart_receive.py :uart_receive.py

## 5.3 UART Wiring (GPIO)

| Pi GPIO Pin | ESP32-S3 Pin | Signal |
| --- | --- | --- |
| GPIO14 (TXD) | RX (GPIO44) | Pi transmits, ESP32 receives |
| GPIO15 (RXD) | TX (GPIO43) | ESP32 transmits, Pi receives |
| GND | GND | Common ground required |

# 6. Deploy Antidote Software

## 6.1 Create Directory Structure

mkdir -p /home/kali/antidote/src

mkdir -p /home/kali/.antidote

## 6.2 Copy Pi Source Files

Extract the pi/ folder from the release zip and copy all .py files:

scp pi/*.py kali@<Pi-IP>:~/antidote/src/

scp -r pi/src/data/ kali@<Pi-IP>:~/antidote/src/

The data/ directory contains company_ids.json (Bluetooth SIG company identifier database, 3973 entries). It must be present at src/data/company_ids.json for manufacturer data lookup to work.

## 6.3 Install systemd Service

sudo cp antidote.service /etc/systemd/system/

sudo systemctl daemon-reload

sudo systemctl enable antidote.service

sudo systemctl start antidote.service

The v1.7 antidote.service uses /home/kali/antidote-env/bin/python (the virtual environment) rather than /usr/bin/python3. This is required — bleak and scapy are installed in the venv only. Ensure the venv was created and populated in section 3.3 before starting the service.

## 6.4 Verify Startup

sudo journalctl -u antidote.service -f

Expected startup sequence in log:

- [WIFI] Monitor interface wlan1mon ready (iw, persistent)

- [UART] Sync loop started

- [BLE] BLE inhale cycle starting

- Inhale complete. Pool=N | WiFi frames=N | BLE=N

## 6.5 Set Interface Configuration

On first run, config.json does not yet exist — it is created automatically when you save settings from the web UI. Do not run the sed command below on a fresh install. Instead, use the web UI to set the WiFi capture interface:

1. Open http://<Pi-IP>:5000 in a browser (or http://192.168.4.1:5000 via the management AP).

2. In the Collection section, set WiFi Capture Interface to wlan1.

3. Click Save & Apply Config. The service restarts and config.json is created.

Alternatively, after the service has run once and created config.json, you can edit it directly:

sudo sed -i 's/"wifi_interface": "wlan0"/"wifi_interface": "wlan1"/' /home/kali/.antidote/config.json

sudo systemctl restart antidote.service

# 7. Persistent Interface Naming (udev)

To ensure wlan1 and wlan2 always refer to the same physical dongles after reboot, pin them by MAC address:

ip link show wlan1 | grep ether  # note the MAC

ip link show wlan2 | grep ether  # note the MAC

Create udev rules file:

sudo nano /etc/udev/rules.d/70-antidote-wifi.rules

Add (substitute your actual MAC addresses):

SUBSYSTEM=="net", ACTION=="add", ATTR{address}=="aa:bb:cc:dd:ee:ff", NAME="wlan1"

SUBSYSTEM=="net", ACTION=="add", ATTR{address}=="11:22:33:44:55:66", NAME="wlan2"

sudo udevadm control --reload-rules

udev rules take effect on next plug or reboot. Verify with iwconfig after reboot.

# 8. Management Access

The Pi Zero 2W has no Ethernet port. Primary management is via the Antidote management AP (wlan0). Connect your laptop or phone to the “Antidote” SSID, then open http://192.168.4.1:5000 in a browser. SSH is also available at the same IP once connected to the AP:

ssh kali@192.168.4.1

Alternatively, connect the Zero 2W to your home WiFi network via NetworkManager so it gets a LAN IP, then access it the same way as the Pi 4B:

sudo nmcli dev wifi connect “YourSSID” password “YourPassword” ifname wlan0

Note: wlan0 is used for the management AP when AP mode is enabled. If you connect wlan0 to your home network instead, the AP will not be available simultaneously. The AP is the recommended approach for portable/field use; WiFi client mode is useful during initial setup at a fixed location.

## 8.1 Enable UART for ESP32-S3 (Zero 2W)

The Pi Zero 2W does not expose /dev/serial0 by default. The primary UART is assigned to the built-in Bluetooth controller. To free it for the ESP32-S3 UART connection, add these lines to /boot/firmware/config.txt:

sudo nano /boot/firmware/config.txt

Add at the end of the file:

enable_uart=1

dtoverlay=disable-bt

enable_uart=1 activates the GPIO UART pins (GPIO14/15). dtoverlay=disable-bt moves Bluetooth off the PL011 UART entirely, freeing /dev/serial0 (an alias for /dev/ttyAMA0) for the ESP32-S3. The Laird BL654 connects via USB as hci0 and is unaffected. Do not add dtoverlay=miniuart-bt — this causes BCM43430 baud rate failures and can break wlan0 on the Zero 2W.

Then disable the built-in Bluetooth service (not needed — the Laird BL654 provides hci0):

sudo systemctl disable hciuart && sudo systemctl stop hciuart

sudo reboot

After reboot, verify /dev/serial0 is present:

ls -la /dev/serial0

Expected output: /dev/serial0 -> ttyAMA0. The Antidote service log will then show [UART] Opened /dev/serial0 at 115200 baud instead of the cannot open warning. Pi 4B users do not need this step — the 4B exposes serial0 without config.txt changes.

## 8.2 wlan0 Boot Persistence (Zero 2W)

On the Zero 2W, systemd-networkd may claim wlan0 via a netplan-generated config at boot, preventing NetworkManager from managing it. Symptom: nmcli device set wlan0 managed yes connects successfully by hand, but wlan0 reverts to unmanaged on the next reboot.

Diagnose with:

sudo journalctl -b | grep -i "wlan0\|netplan\|networkd" | head -10

If you see systemd-networkd: wlan0: Configuring with /run/systemd/network/10-netplan-wlan0.network, the fix is to remove wlan0 from the netplan configuration so NetworkManager owns it exclusively:

ls /etc/netplan/

sudo nano /etc/netplan/<filename>.yaml

Remove or comment out any wlan0 section from the yaml file. Then apply and reconnect:

sudo netplan apply

sudo nmcli device set wlan0 managed yes

sudo nmcli connection up "YourSSID" ifname wlan0

After netplan apply and reboot, wlan0 should connect automatically at boot via NetworkManager without the manual managed yes step. Verify with nmcli device status after reboot — wlan0 should show connected. Pi 4B users do not need this step.

# 9. Troubleshooting

| Symptom | Fix |
| --- | --- |
| wlan1mon did not come UP | Check wifi_interface in /home/kali/.antidote/config.json is set to wlan1, not wlan0 |
| hci0 not found | sudo rfkill unblock bluetooth && sudo hciconfig hci0 up |
| Web UI shows Loading | Hard refresh (Ctrl+Shift+R). If still failing, curl http://<IP>:5000/api/status to verify API is responding |
| ModuleNotFoundError: hardware | Wrong main.py deployed. Ensure pi/main.py (not esp32/main.py) is in ~/antidote/src/ |
| UART not connecting | Check /dev/serial0 exists. Verify GPIO14/15 wiring and common GND. Check uart_port in config. |
| Pool not growing | BLE only works if hci0 is up. WiFi only works with wlan1mon in monitor mode. Check both in logs. |