ANTIDOTE

Installation Guide  |  v1.5  |  From Bare Parts

# 1. Bill of Materials

| Component | Model | Notes |
| --- | --- | --- |
| Single-board computer | Raspberry Pi 4B (2GB+) | Running Kali Linux |
| WiFi dongle x2 | Panda PAU0A AC600 | MT7610U chipset. One for capture, one for AP/inject |
| BLE dongle | Laird BL654 (nRF52840) | Flashed with Zephyr hci_usb firmware |
| Microcontroller | Seeed XIAO ESP32-S3 | Connected via UART (GPIO14/15) and USB |
| MicroSD card | 16GB+ Class 10 | For Kali Linux OS |
| Power supply | 5V 3A USB-C | Official Pi 4 PSU recommended |

# 2. Interface Assignment

| Hardware | Interface | Role | Direction | Description |
| --- | --- | --- | --- | --- |
| Pi Built-in BCM43430 | wlan0 | Idle / Future WiFi uplink | — | Reserved for wireless internet access. Not used for capture or injection. |
| Panda PAU0A #1 (MT7610U) | wlan1 / wlan1mon | WiFi Capture (Passive) | INBOUND | Monitor mode via iw. Passively captures probe requests, beacon SSIDs, and BSSIDs from nearby devices. |
| Panda PAU0A #2 (MT7610U) | wlan2 | SSID Beacon Injection + Management AP | OUTBOUND / MGMT | Injects spoofed beacon frames from pool SSIDs. Also hosts management AP (hostapd) for wireless web UI access. |
| Laird BL654 nRF52840 | hci0 | BLE MAC Spoofing | OUTBOUND | Cycles random MAC address via HCI LE Set Random Address. Re-advertises pool BLE MACs to confuse passive BLE scanners. |
| XIAO ESP32-S3 | /dev/serial0 + /dev/ttyACM0 | BLE Exhale (Advertising) | OUTBOUND | Receives pool samples via UART. Advertises BLE identifiers using onboard radio. Drives RGB LED status indicator. |

wlan0 (Pi built-in BCM43430) cannot use monitor mode on kernel 6.12.x. All capture uses wlan1 (PAU0A #1, MT7610U) via iw monitor mode.

# 3. Raspberry Pi Setup

## 3.1 Install Kali Linux

- Download Kali Linux for Raspberry Pi from kali.org/get-kali

- Flash to MicroSD using Raspberry Pi Imager or balenaEtcher

- Boot Pi, complete initial setup, update: sudo apt update && sudo apt full-upgrade -y

## 3.2 Install System Dependencies

sudo apt install -y python3-pip python3-venv aircrack-ng hostapd dnsmasq

sudo apt install -y bluetooth bluez libbluetooth-dev

sudo apt install -y git iw wireless-tools

Prevent hostapd and dnsmasq from starting as system services (Antidote manages them):

sudo systemctl disable hostapd dnsmasq

sudo systemctl stop hostapd dnsmasq

## 3.3 Create Python Virtual Environment

python3 -m venv /home/kali/antidote-env

source /home/kali/antidote-env/bin/activate

pip install scapy flask pyserial bleak

## 3.4 USB Port Layout

| USB Port | Device | Notes |
| --- | --- | --- |
| USB 3.0 (blue) #1 | PAU0A #1 (wlan1) | WiFi capture dongle |
| USB 3.0 (blue) #2 | PAU0A #2 (wlan2) | AP / injection dongle |
| USB 2.0 #1 | XIAO ESP32-S3 | USB black port on XIAO |
| USB 2.0 #2 | Laird BL654 (hci0) | nRF52840 BLE dongle |

# 4. nRF52840 / Laird BL654 Firmware

## 4.1 Build Zephyr hci_usb Firmware (Windows)

- Install nRF Connect SDK 2.9.2 via nRF Connect for Desktop Toolchain Manager

- Open Toolchain Manager, click the arrow next to v2.9.2, select Open Terminal

- In the terminal, navigate to the Zephyr directory:

cd C:\ncs\v2.9.2\zephyr

- Build the hci_usb sample:

west build -p always -b nrf52840dongle/nrf52840 samples/bluetooth/hci_usb

- Output firmware: build\hci_usb\zephyr\merged.hex

## 4.2 Flash Firmware

- Hold the reset button on the BL654 dongle to enter DFU mode (LED blinks red)

- Open nRF Connect Programmer

- Select the dongle device (appears as BOOTLOADER)

- Add HEX file: select merged.hex from the build output

- Click Write — wait for completion

- Dongle reboots and enumerates as USB HCI device (VID:PID 2fe3:000b)

## 4.3 Verify on Pi

hciconfig

Should show hci0 with USB transport. If not visible:

sudo rfkill unblock bluetooth

sudo hciconfig hci0 up

hciconfig shows 00:00:00:00:00:00 for the public address — this is normal for the nRF52840 in hci_usb mode. Use bluetoothctl show to verify the device is present.

# 5. ESP32-S3 Setup

## 5.1 Flash MicroPython

- Download MicroPython for ESP32-S3 from micropython.org

- Flash using esptool: esptool.py --chip esp32s3 --port /dev/ttyACM0 write_flash 0 firmware.bin

## 5.2 Deploy ESP32 Code

- Extract esp32/ folder from the release zip

- Copy files to ESP32 using mpremote or Thonny:

mpremote connect /dev/ttyACM0 cp main.py :main.py

mpremote connect /dev/ttyACM0 cp uart_receive.py :uart_receive.py

## 5.3 UART Wiring (GPIO)

| Pi GPIO Pin | ESP32-S3 Pin | Signal |
| --- | --- | --- |
| GPIO14 (TXD) | RX (GPIO44) | Pi transmits, ESP32 receives |
| GPIO15 (RXD) | TX (GPIO43) | ESP32 transmits, Pi receives |
| GND | GND | Common ground required |

# 6. Deploy Antidote Software

## 6.1 Create Directory Structure

mkdir -p /home/kali/antidote/src

mkdir -p /home/kali/.antidote

## 6.2 Copy Pi Source Files

Extract the pi/ folder from the release zip and copy all .py files:

scp pi/*.py kali@<Pi-IP>:~/antidote/src/

## 6.3 Install systemd Service

sudo cp antidote.service /etc/systemd/system/

sudo systemctl daemon-reload

sudo systemctl enable antidote.service

sudo systemctl start antidote.service

## 6.4 Verify Startup

sudo journalctl -u antidote.service -f

Expected startup sequence in log:

- [WIFI] Monitor interface wlan1mon ready (iw, persistent)

- [UART] Sync loop started

- [BLE] BLE inhale cycle starting

- Inhale complete. Pool=N | WiFi frames=N | BLE=N

## 6.5 Set Interface Configuration

On first run, edit the saved config to ensure wlan1 is used for WiFi capture:

sudo sed -i 's/"wifi_interface": "wlan0"/"wifi_interface": "wlan1"/' /home/kali/.antidote/config.json

sudo systemctl restart antidote.service

# 7. Persistent Interface Naming (udev)

To ensure wlan1 and wlan2 always refer to the same physical dongles after reboot, pin them by MAC address:

ip link show wlan1 | grep ether  # note the MAC

ip link show wlan2 | grep ether  # note the MAC

Create udev rules file:

sudo nano /etc/udev/rules.d/70-antidote-wifi.rules

Add (substitute your actual MAC addresses):

SUBSYSTEM=="net", ACTION=="add", ATTR{address}=="aa:bb:cc:dd:ee:ff", NAME="wlan1"

SUBSYSTEM=="net", ACTION=="add", ATTR{address}=="11:22:33:44:55:66", NAME="wlan2"

sudo udevadm control --reload-rules

udev rules take effect on next plug or reboot. Verify with iwconfig after reboot.

# 8. Static IP Configuration

To prevent eth0 losing its IP during airmon operations, set a static IP in dhcpcd.conf:

sudo nano /etc/dhcpcd.conf

Add at end of file:

interface eth0

static ip_address=10.0.0.222/24

static routers=10.0.0.1

static domain_name_servers=8.8.8.8

# 9. Troubleshooting

| Symptom | Fix |
| --- | --- |
| wlan1mon did not come UP | Check wifi_interface in /home/kali/.antidote/config.json is set to wlan1, not wlan0 |
| hci0 not found | sudo rfkill unblock bluetooth && sudo hciconfig hci0 up |
| Web UI shows Loading | Hard refresh (Ctrl+Shift+R). If still failing, curl http://<IP>:5000/api/status to verify API is responding |
| ModuleNotFoundError: hardware | Wrong main.py deployed. Ensure pi/main.py (not esp32/main.py) is in ~/antidote/src/ |
| UART not connecting | Check /dev/serial0 exists. Verify GPIO14/15 wiring and common GND. Check uart_port in config. |
| Pool not growing | BLE only works if hci0 is up. WiFi only works with wlan1mon in monitor mode. Check both in logs. |