ANTIDOTE

Privacy Obfuscation Device  |  v1.5

Antidote is a portable privacy device that passively collects Bluetooth and WiFi identifiers broadcast by nearby devices, then re-broadcasts them — creating a cloud of plausible-deniability noise that makes passive surveillance tracking significantly harder.

This file is part of Antidote. Antidote is free software: you can redistribute it and/or modify it under the terms of the GNU General Public License as published by the Free Software Foundation, either version 3 of the License, or (at your option) any later version.

This program is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General Public License for more details.

You should have received a copy of the GNU General Public License along with this program. If not, see <https://www.gnu.org/licenses/>.

## How It Works

Modern phones, laptops, and IoT devices continuously broadcast identifiers: WiFi probe requests (SSIDs your device has connected to), Bluetooth MAC addresses, BLE service UUIDs, and device names. Passive surveillance systems collect and correlate these to track individuals across locations and time.

Antidote intercepts these broadcasts from devices around you, pools them, and re-transmits them — injecting your environment's own identifiers back into the RF space. A surveillance system sees a large number of devices with overlapping, constantly-changing identifiers, making reliable individual tracking impractical.

## What It Does

- Passively captures WiFi probe SSIDs, BSSIDs, and Bluetooth MACs/UUIDs from nearby devices

- Re-injects captured SSIDs as spoofed WiFi beacon frames on a separate radio

- Cycles BLE MAC addresses on a dedicated nRF52840 dongle using confirmed HCI commands

- Pushes pool samples to an ESP32-S3 for further BLE re-advertisement

- Provides a local web UI for monitoring pool status and enabling Phase B features

## Hardware

| Hardware | Interface | Role | Direction | Description |
| --- | --- | --- | --- | --- |
| Pi Built-in BCM43430 | wlan0 | Idle / Future WiFi uplink | — | Reserved for wireless internet access. Not used for capture or injection. |
| Panda PAU0A #1 (MT7610U) | wlan1 / wlan1mon | WiFi Capture (Passive) | INBOUND | Monitor mode via iw. Passively captures probe requests, beacon SSIDs, and BSSIDs from nearby devices. |
| Panda PAU0A #2 (MT7610U) | wlan2 | SSID Beacon Injection + Management AP | OUTBOUND / MGMT | Injects spoofed beacon frames from pool SSIDs. Also hosts management AP (hostapd) for wireless web UI access. |
| Laird BL654 nRF52840 | hci0 | BLE MAC Spoofing | OUTBOUND | Cycles random MAC address via HCI LE Set Random Address. Re-advertises pool BLE MACs to confuse passive BLE scanners. |
| XIAO ESP32-S3 | /dev/serial0 + /dev/ttyACM0 | BLE Exhale (Advertising) | OUTBOUND | Receives pool samples via UART. Advertises BLE identifiers using onboard radio. Drives RGB LED status indicator. |

## Phase A — Core (Always Running)

- WiFi capture on wlan1mon (PAU0A #1, MT7610U)

- BLE scan on hci0 (Laird BL654 nRF52840)

- UART push to ESP32-S3 every 30 seconds

- Web UI at http://<Pi-IP>:5000

## Phase B — Obfuscation (Enable via Web UI)

- SSID Beacon Injection — broadcasts captured SSIDs as fake AP beacons

- BLE MAC Spoofing — cycles MAC on hci0 using pool BLE addresses

- Management AP — wlan2 hosts a WPA2 AP for wireless web UI access

## Current Status (v1.5)

- Phase A: Fully operational

- Phase B: Code complete, ready to enable via web UI

- Web UI: Populating correctly, all controls functional

- WiFi capture: wlan1/MT7610U via iw monitor mode (nexmon not required)

wlan0 (Pi built-in BCM43430) cannot use monitor mode on kernel 6.12.x due to nexmon firmware incompatibility. Capture is delegated to the PAU0A dongle on wlan1.

## Quick Start

- Power on Pi — antidote.service starts automatically

- Open http://<Pi-IP>:5000 in a browser

- Confirm pool is filling (BLE MACs, SSIDs visible in Pool section)

- Enable Phase B features in order: Management AP → SSID Injection → BLE MAC Spoof