ANTIDOTE

Test & Verification Procedures  |  v1.5

This document covers verification of all Antidote features. Tests are ordered from Phase A (always-on) through Phase B (requires enabling). Each section includes setup, test commands, expected log output, and pass criteria.

Pre-requisite: Deploy antidote-v1.5-patch-pretest.zip before testing Phase B. This patch moves SSID injection to wlan2 and removes the airmon-ng dependency from the inject engine.

# Pre-Test Setup

## Apply Pre-Test Patch

- Copy patched files to Pi:

scp pi/ssid_inject.py pi/config.py kali@10.0.0.222:~/antidote/src/

- Update saved config to set ssid_inject_interface to wlan2:

sudo sed -i 's/"ssid_inject_interface": "wlan1"/"ssid_inject_interface": "wlan2"/' /home/kali/.antidote/config.json

- Restart service:

sudo systemctl restart antidote.service

sudo journalctl -u antidote.service -f

## Baseline Interface State

Before testing, confirm interface state:

iwconfig

Expected state at service startup:

- wlan0: Mode:Managed (idle)

- wlan1: physical present; wlan1mon created in Mode:Monitor

- wlan2: Mode:Managed (idle, will be used for AP and/or injection)

- hci0: present (Laird BL654 dongle)

# Test 1 — Phase A Core (Always-On)

## 1.1 Service Startup

Verify the service starts cleanly and all Phase A components initialise:

sudo journalctl -u antidote.service -f

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| wlan1mon created | [WIFI] Monitor interface wlan1mon ready (iw, persistent) | Must appear within 15s of start |
| UART started | [UART] Sync loop started | Appears immediately after monitor setup |
| Phase B engines idle | [SSID] Injection disabled / [BLE-SPOOF] Disabled / [AP] disabled | All three must be disabled on clean start |
| Startup inhale fires | === Inhale cycle start === | Fires within 1s of UART start |

## 1.2 WiFi Capture

Verify WiFi frames are being captured from nearby devices:

sudo journalctl -u antidote.service -f | grep WIFI

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| WiFi cycle completes | [WIFI] Cycle complete. Frames=N | Frames > 0 if any WiFi devices nearby |
| Pool grows with SSIDs | Pool=N ¦ WiFi frames=N | SSIDs, probes, BSSIDs in pool stats |
| No wlan0mon errors | No [WIFI] wlan0mon errors | wlan0 errors no longer expected — confirm absent |

Verify capture directly on wlan1mon:

sudo tcpdump -i wlan1mon -e type mgt 2>/dev/null | head -20

Should show 802.11 management frames (probe requests, beacons) from surrounding devices.

## 1.3 BLE Capture

Verify BLE advertisements are being captured:

sudo journalctl -u antidote.service -f | grep BLE

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| BLE cycle completes | [BLE] Cycle complete. Advertisements seen=N | N > 0 if any BLE devices nearby |
| BLE MACs captured | BLE MACs=N | Should see phones, headphones, etc. |
| UUIDs captured | UUIDs=N | Service UUIDs from BLE advertisements |

## 1.4 UART Push to ESP32

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| UART port opens | [UART] Opened /dev/serial0 at 115200 | Within first 30s of service start |
| Regular pushes | [UART] Pushed N identifiers to ESP32-S3 | Every 30s, N should match pool size |
| No UART errors | No [UART] error messages | Persistent errors indicate wiring issue |

If UART does not open, check: (1) GPIO14/15 wiring and common GND, (2) ESP32 is powered via USB, (3) uart_port config is /dev/serial0.

## 1.5 Web UI

Navigate to http://10.0.0.222:5000 (substitute Pi IP):

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| Page loads | UI renders, no blank page | Hard refresh (Ctrl+Shift+R) if cached |
| Pool stats populate | BLE MACs, SSIDs show counts | Non-zero values after first inhale |
| Freshness indicators | Green or yellow colour bands | Stale/red only if no inhale recently |
| Log panel populates | Timestamped log lines visible | Most recent 60 lines displayed |
| Config fields show | All fields populated with values | No empty or Loading fields |
| API direct check | curl .../api/status returns JSON | Run: curl -s http://10.0.0.222:5000/api/status ¦ python3 -m json.tool |

# Test 2 — Management AP (wlan2)

## 2.1 Enable AP

- In web UI: check Enable Management AP

- Set SSID: Antidote, Password: SnoopThem, Channel: 1

- Click Save, then click Restart AP

Watch log for:

sudo journalctl -u antidote.service -f | grep AP

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| AP starts | [AP] Management AP 'Antidote' started on wlan2 | Within 5s of Restart AP |
| Web UI accessible | [AP] Web UI accessible at http://192.168.4.1:5000 | Confirm in log |
| wlan2 gets IP | ip addr show wlan2 shows 192.168.4.1 | Run: ip addr show wlan2 |
| hostapd running | hostapd process active | Run: pgrep -a hostapd |
| dnsmasq running | dnsmasq process active | Run: pgrep -a dnsmasq |

## 2.2 Connect a Client Device

- On a phone or laptop, scan for WiFi networks

- Connect to SSID: Antidote, Password: SnoopThem

- Open browser and navigate to http://192.168.4.1:5000

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| SSID visible | Antidote appears in WiFi scan | May take 5-10s to appear |
| Client connects | Device gets IP in 192.168.4.x range | DHCP pool: .10 to .50 |
| Web UI loads over AP | Full UI loads at 192.168.4.1:5000 | Confirms hostapd + dnsmasq + Flask all working |

## 2.3 Verify No Conflict with Capture

While AP is active, trigger an inhale from the web UI and confirm WiFi capture still works on wlan1:

sudo journalctl -u antidote.service -f | grep -E 'WIFI|AP'

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| Capture unaffected | [WIFI] Cycle complete. Frames=N | wlan1mon capture continues independently |
| AP stays up | [AP] log shows no errors | No restarts or failures during inhale |

# Test 3 — SSID Beacon Injection (wlan2)

SSID injection now uses wlan2. If Management AP is enabled on wlan2, disable it before testing injection — they cannot share the same interface simultaneously.

## 3.1 Pre-Injection Check

Ensure pool has SSIDs to inject:

curl -s http://10.0.0.222:5000/api/status | python3 -m json.tool | grep wifi_ssid

wifi_ssid count should be > 0. If zero, trigger an inhale and wait for WiFi cycle to complete.

Confirm wlan2 is not in monitor mode and not running as AP:

iwconfig wlan2

pgrep hostapd && echo 'AP running - disable first' || echo 'AP not running - OK'

## 3.2 Enable Injection

- In web UI: confirm ssid_inject_interface is wlan2

- Check Enable SSID Beacon Injection

- Set rate: 5, channels: 1,6,11

- Click Save

sudo journalctl -u antidote.service -f | grep SSID

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| Monitor created | [SSID] Monitor interface wlan2mon ready | wlan2mon created via iw |
| Injection starts | [SSID] Injecting on wlan2mon, channels=[1,6,11] | Confirms engine running |
| Beacons sent | [SSID] Injected N beacons ch1/6/11 | Debug log — N > 0 |

## 3.3 Verify Beacons on Air

On a second device or using wlan1mon (capture interface), scan for injected SSIDs:

sudo tcpdump -i wlan1mon -e type mgt subtype beacon 2>/dev/null | head -30

Alternatively, use a WiFi scanner app on a phone to see if pool SSIDs appear as nearby networks.

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| Beacons visible on wlan1mon | Beacon frames with pool SSIDs visible | Self-filter should block OWN BSSIDs from pool |
| SSIDs visible on phone scanner | Pool SSIDs appear as nearby APs | Confirms over-air transmission working |
| Self-filter working | Injected BSSIDs not added to pool | Pool BSSID count should not grow from own beacons |

To confirm self-filter: note the BSSIDs assigned to injected SSIDs in the log, then check they do not appear in the web UI pool detail view.

# Test 4 — BLE MAC Spoofing (hci0)

## 4.1 Pre-Spoof Check

Confirm hci0 is up and the Laird BL654 dongle is present:

hciconfig

sudo rfkill unblock bluetooth && sudo hciconfig hci0 up

Confirm pool has BLE MACs to spoof:

curl -s http://10.0.0.222:5000/api/status | python3 -m json.tool | grep ble_mac

## 4.2 Enable MAC Spoofing

- In web UI: verify ble_mac_spoof_interface is hci0

- Check Enable BLE MAC Spoofing

- Set interval: 30 seconds

- Click Save

sudo journalctl -u antidote.service -f | grep BLE-SPOOF

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| Spoofer starts | [BLE-SPOOF] MAC spoofing engine started | Appears within 2s of Save |
| First MAC set | [BLE-SPOOF] Broadcasting as XX:XX:XX:XX:XX:XX | MAC from pool |
| MAC cycles | New MAC every 30s in log | Confirms cycling working |

## 4.3 Verify MAC Change

Confirm the HCI random address is actually changing:

bluetoothctl show | grep 'Address'

Run this twice, 30 seconds apart — address should change between readings.

To manually verify HCI command is working:

sudo hcitool -i hci0 cmd 0x08 0x0005 DE AD BE EF CA FE

bluetoothctl show | grep Address

Address should reflect DE:AD:BE:EF:CA:FE (reversed byte order) — confirming HCI LE Set Random Address works.

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| MAC changes on cycle | Different address every 30s | Confirmed via bluetoothctl show |
| HCI command accepted | Status 0x00 in hcitool output | 0x00 = success |
| Cycling from pool | Addresses match pool BLE MACs | Confirms pool integration |

## 4.4 Verify BLE Advertisement Visible

Use a BLE scanner app (nRF Connect, LightBlue, etc.) on a phone to confirm the Pi is advertising:

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| Device visible in scanner | BLE device with pool MAC visible | Name may show as blank or generic |
| MAC rotates in scanner | Different address appears after interval | May need to refresh scanner |

# Test 5 — Combined Operation

## 5.1 Full Phase B Active

Enable all Phase B features simultaneously (note: AP and SSID injection share wlan2 — test one at a time) and verify Phase A continues normally:

- Enable Management AP on wlan2 — verify AP accessible

- Trigger manual inhale — confirm wlan1mon capture still works

- Disable AP, enable SSID injection on wlan2 — verify beacons on air

- Enable BLE MAC spoofing on hci0 — verify cycling

- Trigger another inhale — confirm BLE capture continues alongside spoofing

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| Capture unaffected by spoof | BLE MACs still growing in pool | Inhale captures new devices while spoofing |
| WiFi capture unaffected | wlan1mon frames > 0 each cycle | wlan2 injection doesn't impact wlan1 capture |
| UART pushes continue | Push every 30s throughout | ESP32 receiving updates during all Phase B activity |
| No service crashes | Service stays running for 30+ mins | No Restart entries in journalctl |

## 5.2 Pool Age-Out

Verify the age-out mechanism works (requires patience — default 2 hours):

To accelerate test: temporarily set pool_retention_hours to 0.05 (3 minutes) in web UI, trigger two inhales, then observe pool shrinking after 3 minutes with no new inhale.

sudo journalctl -u antidote.service -f | grep 'Pool='

| Check | Expected | Pass Criteria / Notes |
| --- | --- | --- |
| Pool shrinks after age-out | Pool=N decreases without inhale | After retention period, old entries purged |
| Freshness goes stale | Web UI shows orange/red freshness | Confirms freshness timer working |

# Known Limitations (v1.5)

| Limitation | Detail / Workaround |
| --- | --- |
| wlan2 shared: AP vs Injection | Management AP and SSID Injection both require wlan2. Only one can be active at a time. Planned fix: dedicated interface separation in v1.6. |
| wlan0 no monitor mode | Pi built-in BCM43430 cannot use monitor mode on kernel 6.12.x due to nexmon firmware incompatibility. Capture uses wlan1 (MT7610U) instead. |
| Web UI wlan dropdowns | Interface dropdown in web UI still shows wlan0 as an option. wlan1/wlan2 must be set manually. Planned fix in next UI update. |
| udev interface pinning | wlan1 and wlan2 may swap on reboot if both PAU0A dongles enumerate simultaneously. Fix: add udev rules pinning by MAC address (see Install Guide). |
| BLE capture during MAC spoof | BLE capture and MAC spoofing both use hci0. The spoofer briefly stops advertising during each MAC change. Minor gap only. |