ANTIDOTE

User Guide  |  v1.6

# Design Intent

Antidote’s design intent is to generate localized electronic identifier traffic that obfuscates location information for those identifiers, without negative impact on surrounding electronic traffic.

Antidote is specifically designed to coexist with legitimate local traffic and to only impact problematic electronic surveillance systems. Only systems attempting to violate the fourth amendment through mass data collection should be materially impacted by its operation. They will simply lose the ability to reliably specify the precise time and location of electronic identifier encounters.

Antidote collects BLE and WiFi identifiers from the surrounding environment and rebroadcasts them as BLE advertisements, creating a cloud of ambient identifiers that obscures the actual device fingerprints of the operator and surrounding devices. Random identifier data would be easily dismissed and would not interfere with broad searches for specific identifiers. By collecting local identifiers before retransmission, the system creates a fog of traffic indistinguishable from data collected from the ‘true’ identifier source. Again, the system is designed specifically to NOT interfere with network or communications traffic. Further, the system makes use of hardware and firmware capabilities specifically designed to enhance privacy by allowing rolling or changing MAC addresses, SSIDs and other identifiers to avoid tracking.

The above statement is intended to provide design intent, and is not a legal opinion. Users of the system should verify the legality of operation in their jurisdiction and take on all responsibility for its operation.

# 1. Device Overview

| Hardware | Interface | Role | Direction | Description |
| --- | --- | --- | --- | --- |
| Pi Built-in BCM43430 | wlan0 | Management AP / Future WiFi uplink | — | Hosts management AP (hostapd) for wireless web UI access. Dedicated interface — no conflict with injection or capture. |
| Panda PAU0A #1 (MT7610U) | wlan1 / wlan1mon | WiFi Capture (Passive) | INBOUND | Monitor mode via iw. Passively captures probe requests, beacon SSIDs, and BSSIDs from nearby devices. |
| Panda PAU0A #2 (MT7610U) | wlan2 | SSID Beacon Injection | OUTBOUND | Injects spoofed beacon frames from pool SSIDs. Dedicated outbound injection interface. |
| Laird BL654 nRF52840 | hci0 | BLE MAC Spoofing | OUTBOUND | Cycles random MAC address via HCI LE Set Random Address. Re-advertises pool BLE MACs to confuse passive BLE scanners. |
| XIAO ESP32-S3 | /dev/serial0 + /dev/ttyACM0 | BLE Exhale (Advertising) | OUTBOUND | Receives pool samples via UART. Advertises BLE identifiers using onboard radio. Drives RGB LED status indicator. |

## Signal Flow

| Capture |  | Pool |  | Re-broadcast |
| --- | --- | --- | --- | --- |
| wlan1mon: WiFi probes, beacons, BSSIDs | > | Up to 2000 identifiers, 2hr age-out | > | wlan2: SSID beacon injection ¦ hci0: BLE MAC spoofing ¦ ESP32: BLE advertising |
| hci0: BLE MACs, UUIDs, device names, manufacturers |  |  |  |  |

# 2. Web Interface

## 2.1 Accessing the UI

- Wired/LAN: http://<Pi-IP>:5000 (default 10.0.0.222:5000)

- Wireless: Enable Management AP first, then http://192.168.4.1:5000

## 2.2 Status Panel

The top section shows real-time pool statistics refreshed every 8 seconds:

| Field | Description |
| --- | --- |
| Total | Total identifiers currently in pool |
| Probe MACs | WiFi probe request source MACs captured |
| BLE MACs | Bluetooth device addresses captured |
| BLE UUIDs | BLE service UUIDs from advertisements |
| SSIDs | WiFi network names from beacons and probes |
| BSSIDs | WiFi access point hardware addresses |
| Freshness | Color-coded: green = <5min, yellow = <15min, orange = <1hr, red = stale |

## 2.3 Inhale Controls

- Inhale Frequency — how often the capture cycle runs (seconds). Set to 0 for continuous.

- Inhale Duration — how long each WiFi capture window stays open.

- Trigger Inhale Now — immediately starts a capture cycle regardless of schedule.

## 2.4 Pool Settings

- Max Pool Size — maximum number of identifiers stored (default 2000).

- Pool Retention Hours — how long before old identifiers are aged out (default 2 hours).

# 3. Phase B Features

All Phase B features are disabled by default. Enable them in the order shown below.

## 3.1 Management AP

Hosts a WPA2 WiFi access point on wlan2, enabling wireless access to the web UI.

- Check Enable Management AP

- Set SSID (default: Antidote) and Password (default: SnoopThem)

- Set Channel (default: 1)

- Click Save, then click Restart AP

- Connect a device to the Antidote WiFi network

- Access web UI at http://192.168.4.1:5000

## 3.2 SSID Beacon Injection

Broadcasts captured WiFi SSIDs as fake beacon frames on wlan2, making surveillance systems see many phantom access points.

- Ensure pool contains WiFi SSIDs (check SSID count in status panel)

- Check Enable SSID Beacon Injection

- Verify interface is wlan1 (capture) — injection shares the MT7610U

- Set injection rate (default: 5 beacons/sec) and channels

- Click Save — log should show [SSID] Injection engine started

SSID injection and WiFi capture currently both use wlan1. In a future update these will be separated to wlan1 (capture only) and wlan2 (inject only).

## 3.3 BLE MAC Spoofing

Cycles the BLE MAC address on the nRF52840 dongle (hci0) using pool BLE addresses, causing passive BLE scanners to see many different devices.

- Ensure Laird BL654 dongle is connected and hci0 is visible (hciconfig)

- Check Enable BLE MAC Spoofing

- Verify interface is hci0

- Set spoof interval (default: 30 seconds between MAC changes)

- Click Save — log should show [BLE-SPOOF] MAC spoofing engine started

# 4. Monitoring

## 4.1 Service Log

View live log output from the service log panel in the web UI, or on the Pi:

sudo journalctl -u antidote.service -f

## 4.2 Key Log Messages

| Message | Meaning |
| --- | --- |
| [WIFI] Monitor interface wlan1mon ready | WiFi capture is running correctly |
| [BLE] Cycle complete. MACs=N | BLE scan completed, N devices found |
| [UART] Pushed N identifiers | Pool sample sent to ESP32-S3 successfully |
| [SSID] Injecting on wlan1mon | SSID beacon injection active |
| [BLE-SPOOF] Broadcasting as XX:XX | MAC cycling active on hci0 |
| [AP] Management AP started | wlan2 AP is up, 192.168.4.1 accessible |
| [WIFI] Monitor interface missing | wlan1mon dropped — recovery will be attempted next cycle |

## 4.3 Pool Health

A healthy pool shows:

- BLE MACs growing during and after inhale cycles

- WiFi SSIDs/Probes growing if devices with WiFi are nearby

- Freshness indicators green or yellow for recently-seen categories

- UART pushes every 30 seconds confirming ESP32 is connected

# 5. Config Reference

| Key | Default | Description |
| --- | --- | --- |
| wifi_interface | wlan1 | Physical interface for WiFi capture |
| inhale_frequency | 300 | Seconds between inhale cycles (0 = continuous) |
| inhale_duration | 60 | Seconds per WiFi capture window |
| max_pool_size | 2000 | Maximum identifiers in pool |
| pool_retention_hours | 2 | Hours before old identifiers age out |
| ssid_inject_enabled | false | Enable/disable SSID beacon injection |
| ssid_inject_interface | wlan1 | Interface for injection (shared with capture currently) |
| ssid_inject_rate | 5 | Beacons per second |
| ssid_inject_channels | 1,6,11 | Channels to rotate through during injection |
| ble_mac_spoof_enabled | false | Enable/disable BLE MAC cycling |
| ble_mac_spoof_interval | 30 | Seconds between MAC address changes |
| mgmt_ap_enabled | false | Enable/disable management AP on wlan2 |
| mgmt_ap_ssid | Antidote | SSID for management AP |
| mgmt_ap_password | SnoopThem | WPA2 password for management AP |
| uart_port | /dev/serial0 | Serial port for ESP32-S3 communication |
| uart_sync_interval | 30 | Seconds between UART pushes to ESP32 |

# 5. Web UI Controls Reference

The web UI is accessible at http://10.0.0.222:5000 over Ethernet or http://192.168.4.1:5000 when connected to the management AP. All settings take effect when Save & Apply Config is clicked. Phase B features require explicit enable after saving.

## 5.1 System Status Card

Mode badge — Shows IDLE, INHALE, EXHALE, or ERROR. INHALE is active during a collection cycle. IDLE is normal operating state between cycles.

Pool statistics tiles — Eight tiles show counts by category. The colored dot in the top-right corner of each tile indicates freshness: green = seen within 5 min, yellow = within 15 min, amber = within 1 hour, red = older. Counts drop when identifiers age out of the retention window.

ESP32 UART row — Green dot = UART connection active and pushing. Red = UART disconnected or error. Shows total push count and last error if any.

Show pool sample (5 per category) — Checking this box expands a grid below the status card showing up to 5 randomly selected identifiers per category. Refreshes each time the status poll runs (every 8 seconds). Each category card also shows a Full Pool button — clicking it opens a modal snapshot of the entire pool for that category, sorted alphabetically. The snapshot is taken at the moment of the button click and is not live-updated.

## 5.2 Collection (Inhale) Card

Duration (sec) — How long each inhale cycle runs. 60 seconds is the default. Set to 0 for continuous (runs until manually stopped).

Frequency (sec between cycles) — Time between the end of one inhale cycle and the start of the next. Default 300 seconds (5 minutes). Lower values keep the pool fresher at the cost of more radio activity on wlan1.

Collect checkboxes — Six toggles control what data types are added to the pool during each inhale cycle. BLE MAC Addresses are the primary spoofing source. WiFi Beacons / SSIDs feed the SSID injection engine. Disabling a type stops it being collected but does not remove existing entries from the pool.

WiFi Capture Interface — Must be set to wlan1 (MT7610U PAU0A dongle #1). wlan0 (Pi built-in BCM43430) cannot do monitor mode on kernel 6.12.x and must not be used here.

WiFi Starting Channel — The channel wlan1mon begins scanning on. The inhale engine hops channels automatically. Default 6.

## 5.3 Pool Settings Card

Max Pool Size — Maximum number of unique identifiers held in memory across all categories. Default 2000. When the pool is full, the least-recently-seen identifier is evicted to make room.

Pool Retention (hours) — Identifiers not seen within this window are automatically removed from the pool. Default 2 hours. Lower values keep the pool representing only currently-nearby devices; higher values create a larger obfuscation cloud from devices seen earlier.

Exhale / UART Sync Interval (sec) — How often the Pi automatically pushes a fresh sample to the ESP32-S3 over UART. Default 30 seconds. Lower values mean the ESP32 rotates through more identifiers per minute; higher values reduce serial bus activity.

## 5.4 ESP32-S3 UART Sync Card

Serial Port — UART device path. Default /dev/serial0 (GPIO pins 14/15). Alternatively /dev/ttyACM0 if connected via USB black port.

Sample Size (IDs per push) — Number of randomly selected identifiers sent to the ESP32 on each UART push. Default 50. Each push replaces the ESP32’s working set entirely. Higher values give the ESP32 more variety to cycle through between pushes.

## 5.5 SSID Injection Card (Phase B)

Enable beacon injection — Starts the SSID injection engine on wlan2. The engine places wlan2 into monitor mode (wlan2mon) and transmits 802.11 beacon frames for each SSID in the pool, cycling through channels 1, 6, and 11. Each SSID is assigned a unique random BSSID that is stable for the session, so passive scanners see consistent ghost access points. Checking this box and saving starts the engine immediately; unchecking stops it.

Inject Interface — Must be set to wlan2 (PAU0A dongle #2, MT7610U). The engine creates wlan2mon automatically.

Beacons per second per SSID — Injection rate. Default 5 beacons/second/SSID. Lower values reduce RF load; higher values increase the likelihood passive scanners capture each SSID on a given channel pass.

Show last injected SSID — Optional display showing the most recently injected SSID and its associated BSSID, plus a running beacon count. Polled every 8 seconds alongside the main status update. Useful for confirming injection is running and cycling through pool entries. Disable when not needed to reduce unnecessary API calls.

## 5.6 BLE MAC Spoofing Card (Phase B)

Enable MAC spoofing — Starts the BLE MAC spoofing engine on hci0 (Laird BL654 nRF52840 dongle). On each cycle, the engine picks a random BLE MAC address from the pool and sets it as the LE random address via direct HCI command (OCF 0x0005), then enables non-connectable LE advertising so other devices observe the spoofed address. Checking and saving starts the engine immediately; unchecking stops it.

HCI Interface — Must be hci0. The Pi’s built-in Bluetooth must be disabled in /boot/config.txt (dtoverlay=disable-bt) so that hci0 maps to the nRF52840 dongle running Zephyr hci_usb firmware, not the Pi’s own controller.

MAC cycle interval (sec) — How often the broadcast MAC address changes. Default 30 seconds. Shorter intervals cycle through more identifiers faster; longer intervals give passive scanners more time to record each address.

Show last broadcast MAC — Optional display showing the MAC address the engine last set, the cycle count, and the HCI confirm status. The “HCI confirm” field shows ✓ HCI accepted (green) when both the LE Set Random Address command (OCF 0x0005) and the subsequent LE advertising enable were accepted by the nRF52840 controller, or ✗ HCI rejected (red) if either step failed. Note: the nRF52840 Zephyr hci_usb firmware does not expose a read-back command for the LE random address, so confirmation is based on the HCI command return status rather than a readback. Disable when not needed.

## 5.7 Management AP Card (Phase B)

Enable AP — Starts hostapd and dnsmasq on wlan0 (Pi built-in BCM43430), creating a WPA2 WiFi access point. Once connected, the web UI is accessible at http://192.168.4.1:5000. In v1.6, wlan0 is dedicated to the AP and is completely independent of wlan1 (capture) and wlan2 (injection) — all three can operate simultaneously without conflict.

AP Interface — Must be wlan0. Do not set this to wlan1 or wlan2 as those are dedicated to capture and injection respectively.

Restart AP button — Force-cycles hostapd and dnsmasq without toggling the enable checkbox. Use this if the AP becomes unresponsive without disabling it entirely.

## 5.8 Actions Card

Save & Apply Config — Writes all current form values to /home/kali/.antidote/config.json and immediately applies any enable/disable transitions for Phase B features. This is the primary control action; all other buttons are supplementary.

Force Inhale Cycle — Immediately queues an inhale cycle regardless of the scheduled frequency. Useful for manually refreshing the pool after moving to a new location.

Force UART Push to ESP32 — Immediately sends a fresh random sample to the ESP32-S3 over UART, outside the normal sync schedule. The toast message confirms the push and shows the count sent.

Reset to Defaults — Resets all configuration values to factory defaults and saves immediately. All Phase B features will be disabled. Requires confirmation dialog.